Corrigendum on the paper:

Author: Seiichi Nakamori

“Design of RLS Wiener FIR filter using covariance information in linear discrete-time stochastic systems”, Digital Signal Processing, 20 (2010) pp.1310-1329

| Place | Incorrect expressions | Corrected expressions |
| --- | --- | --- |
| Eq.10: lines 3, 7 | "A(k)" | "B(k)" |
| Eq.(16): Line 2 | "A(k)" | "B(k)" |
| Eq.(30): lines 3, 7 | "Hl4(k,k)" | "HKx(0)l4(k,k)" |
| Eq.(33): lines 1, 4, 8 | "l6(k,k+L)" | "l4(k,k+L)" |
| Eq.(37): line 1 | "Φl4(k,k+L)ΦT" | "(ΦT)-1l4(k,k+L)ΦT" |
| Eq.(37): line 1 | "+Φk+1(ΦT)k+1D1(k+1+L,k+1+L)" | "+D1(k+1+L,k+1+L)" |
| Eq.(37): line 2 | "-Φk+1(ΦT)k+1D2(k+1+L,k)" | "-D2(k+1+L,k)" |
| Eq.(37): line 2 | "l6(k,k+L)" | "l4(k,k+L)" |
| Eq(38): line 2 | "-G1(k+1+L,k)" | "-G2(k+1+L,k)" |
| Eq.(39) |  | Eq.(39) is not necessary. |
| Eq.(41) | "l6(k,k+L)" | "l4(k,k+L)" |
| Eq.(47) | +ΦΦTD3(1+L,1+L) | +D3(1+L,1+L) |
| Eq.(49) |  | Eq.(49) is not necessary. |
| Eq.(A.15): line 2 | "A(k)" | "B(k)" |
| Eq.(A.19): line 1 | "L(k+1+L,k)" | "U(k+1+L,k)" |
| Eq.(A.36): line 3 | "A(k)" | "B(k)" |
| Eq.(A.37): line 3 | "A(k)" | "B(k)" |
| Eq.(B.8): lines 4, 6 | "A(k)" | "B(k)" |
| Eq.(B.8): lines 8, 10 | "Hl4(k,k+L)" | "HKx(0)l4(k,k+L)" |
| Eq.(B.11) | "Φk" | "(ΦT)-k" |
| Eq.(B.18): lines 9, 12, 16 | "l6(k,k+L)" | "l4(k,k+L)" |
| Eq.(B.19) | "l6(k,k+L)" | "l4(k,k+L)" |
| Eq.(B.23): Line 3 | "Φl4(k,k+L)ΦT" | "(ΦT)-1l4(k,k+L)ΦT" |
| Eq.(B.23): Line 3 | "+Φk+1(ΦT)k+1D1(k+1+L,k)" | "+D1(k+1+L,k)" |
| Eq.(B.23): Line 4 | "-Φk+1(ΦT)k+1D2(k+1+L,k)" | "-D2(k+1+L,k)" |
| Eq.(B.23): Line 4 | "l6(k,k+L)" | "l4(k,k+L)" |
| Eq.(B.24): Line 2 | "A(k)" | "B(k)" |
| Eq.(B.24): Line 4 | "-G1(k+1+L,k)" | "-G2(k+1+L,k)" |
| Eq.(B.25), Eq.(B.34), Eq.(B.35). |  | Eq.(B.25) , Eq.(B.34) and Eq.(B.35) must be removed. |
| Eq.(B.38) | "Φq(1,1+L)ΦT(ΦT)L" | "(ΦT)-1q(1,1+L)ΦT(ΦT)L" |
| Eq.(B.38) | "+ΦΦTD3(1+L,1+L)" | +D3(1+L,1+L)" |